| Technical spec / model | ## DVR-8916 |  |  |
| --- | --- | --- | --- |
| Compression | 200-500:1 |  |  |
| Video compression rate | 80-350MB/hour/CH |  |  |
| HDD space consumption | 320x240, 640x320 and 640x480 pixels |  |  |
| Operation system | RTOS |  |  |
| Video input | 1,4,8,12,16 CH |  |  |
| Video output | 2CH AV/VGA |  |  |
| Resource | 400FPS PAL |  |  |
| Recording Speed | 400FPS PAL |  |  |
| Audio input Audio output | 16CH Audio in 1CH Audio out |  |  |
| Image multiplex | 1,4,8,9,16CH |  |  |
| Playback images | 1CH |  |  |
| Monitor resolution | 704X576 (PAL) |  |  |
| Recording resolution | 352X288 (PAL) |  |  |
| Multiplex operation | YES, support monitor, recording, play back, remotely surveillance and backup simultaneously. |  |  |
| Central control | YES (optional), support the central control and monitor towards a series of DVRs through control keyboard or central control software. |  |  |
| Remote surveillance speed 512K(bandwidth) | 1-25 fps/1CH PAL (real time) |  |  |
| Operation interface | Graphic operation interface, friendly interface, easy to learn |  |  |
| CD-RW Backup | YES, |  |  |
| Support PSTN Compatible modems | YES, compatible with Bullet,TP-Link,Start |  |  |
| I E Browser | YES, support remote view of 1, 4, 9, 16 Ch, playback, control and setting |  |  |
| Motion detection | 192 areas/channel |  |  |
| Network | TCP/IP, LAN, WAN, DDN |  |  |
| P/T/Z/ speed dome | YES, support more than 15 brands including Pelco, Lilin etc. |  |  |
| Language | Traditional Chinese,BSimplified Chinese,English,Spanish. |  |  |
| Print | YES, print out recording data in CD through LAN or Internet |  |  |
| HDD Storage | 1-8 pieces of HDD, no capacity limit (5pcs with CDRW installed) |  |  |