| DVR IR 10 | 1/4" Color IR Camera |
| --- | --- |
| Item | Describe |
| Sensor Chip Brand | SONY |
| Picture Element | PAL:500(H)x582(V) |
| Scanning Frequency | CCIR/PAL:15625Hz(H),50Hz(V) |
| H. Resolution | More than 400 TV Lines |
| Illumination | 0 Lux (IR ON) |
| Infrared Ray LED | 12 pcs |
| Infrared Ray Distance | 10 Meters |
| Water Resistance | IP68 |
| Synchronizing | Internal Sync., 2:1 Interlace |
| Auto White Balance | AUTO |
| Auto Gain Control | AUTO |
| S/N Ratio | More than 48 dB |
| Gamma Correction | 0.45 |
| Auto Shutter | AUTO[1/60(50)~1/100,000] |
| Video Output | 1.0Vp-P75ohm |
| Lens Mount | 4.3 mm Board lens |
| Backlight Compensation | AUTO |
| Power | DC 12V / 280mA |
| Dimension / Weight | 53diax85(D)mm / 270g |
| Operating Temperature | -10°C ~ +45°C |