| Effective Pixels | NTSC: 816(H) x 495(V) ; PAL:798(H) x 548(V) |
| --- | --- |
| Scanning Frequency | NTSC: 15,734Hz(H), 60Hz(V); PAL: 15,625Hz(H) , 50Hz(V) |
| Scanning System | NTSC: 525 line 2:1 interlace; PAL: 625 lines 2:1 interlace |
| Horizontal | 480 TV lines |
| BLC | Auto |
| Electronic Shutter | NTSC: 1/60-100,000sec ; PAL:1/50-100,000sec. |
| Gain Control | AGC |
| Gamma Correction | 0.45 |
| Image Device | 1/3" Color Night Vision CCD image sensor |
| IR LED | High-light LE D x 12pcs with condensing lens |
| IR View Angle | View angle: 80°, Beam spread: 20°/per LED |
| IR Wavelength | 850nm |
| Lens | 25mm/ F1.8 |
| Sensitivity | 0.05 Lux , 0 Lux at IR on |
| Signal to Noise | More than 56dB |
| SYNC System | Internal |
| Video Output | Composite 1.0 Vp-p at 75 ohm |
| With Balance | ATW 2000-10000°K |
| Power Supply | 12VDC / 24VAC / 85-260VAC |
| IR Distance | 50 Meters (Based on: object of view reflected IR light) |
| IR Status | Under 10 Lux by CDS auto control |
| LED Life | More than 20,000 hours |
| Protection Class | IP 66 classification |
| Power | IR on: 390mA, IR off: 90mA |
| Construction | Aluminum with sunshield weatherproof |
| Temperature | Operating: -20 to 50°C, Storage: -30 to 60°C |
| Dimensions | 95(Φ) x 200(L) mm |
| Weight | 1.32 Kgs |

DVR IR 100